Alarm:     ID 21: Load Unit Not Found


How To Fix:
This alarm occurs when the box sensor fails to make contact with the platform leg.

To resolve:

- Reposition the platform on the floor so the sensor can make contact with the platform leg.

If repositioning isn’t possible:

- Switch the AGV to Manual Mode and raise the table to its maximum height.

- Return the AGV to Automatic Mode.

- Press 1 on the AGV's front screen to load the platform.

For AGV routing adjustments:

- Cancel the existing mission on the AGV computer.

- Manually re-route the AGV's path.

- Rename the platform at its end destination with the correct number.

If you encounter issues, please contact a qualified AGV Technician.